The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

A high resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at https://www.masterbond.com/newsrelease/ep17hts-da.

-----------------------------------------------------------------

FOR IMMEDIATE RELEASE

Electrically Conductive Die Attach Epoxy with High Glass Transition Temperature

Master Bond EP17HTS-DA is a new one component, no mix, die attach epoxy that is electrically conductive and withstands high temperatures. It is a silver filled system. Typically, glass transition temperature (Tg) declines with the addition of a filler like silver. However, with this specialty formulation, a high Tg of 140-150°C is maintained and it passes MIL-STD-883J thermal stability requirements at 200°C.

Its strength profile is also notably robust with a die shear strength of 35-40 kg-f at room temperature. EP17HTS-DA meets NASA low outgassing specifications and its service temperature extends from -80°F to 550°F. It has a low volume resistivity of less than 0.005 ohm-cm and also performs well as a heat conductor.

EP17HTS-DA is a thixotropic paste and can be easily dispensed manually or via automated dispensing systems. This compound is not premixed and frozen and has an unlimited working life at room temperature. It requires a heat cure of 300°F for 2 to 3 hours, followed by post curing at 350°F for 1 to 2 hours.

This product bonds well to many substrates including metals, ceramics and plastics. It is recommended for applications where low volume resistivity and high temperature resistance is required. EP17HTS-DA is available for use in syringes and glass jars, in sizes ranging from 20 grams to several pounds.

Master Bond Die Attach Adhesives

Master Bond EP17HTS-DA is a one part, electrically conductive epoxy with high glass transition temperature (Tg). It offers a high die shear strength and a convenient dispensing profile. Read more about Master Bond’s adhesives for die attach as well as other demanding microelectronic applications at https://www.masterbond.com/industries/die-attach-epoxy-adhesives  or contact Tech Support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

Note to Editors:

For a full product description, please visit: https://www.masterbond.com/tds/ep17hts-da

Check out new videos on our YouTube channel: http://www.youtube.com/user/MasterBondVideo

You can embed any of our videos on your website.

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #